4350.1 REV-1

CHG-2

____________________________________________________________________

CHAPTER 19:  ENVIRONMENTAL ISSUES

SECTION 1:  LEAD-BASED PAINT

19-1.     INTRODUCTION.  Since the early 1970s the medical

community continues to uncover the dangerous effects

the lead in many of our commonly used substances has on

the population, especially children.  As a result

measures have been taken to remove the source of

contamination from our society through laws and

regulations.  With the introduction of the Lead-Based

Paint Poisoning Prevention Act in the early 1970s and

its subsequent amendments throughout the 1980s,

regulations were developed which required HUD to

"establish procedures to eliminate as far as

practicable the hazards of lead based paint poisoning."

As a consequence of this Act, lead sources such as

leaded gasoline and paint were gradually removed from

use in the United States.  However, the problem of

lead-based paint in existing housing built prior to

1978 still remains.  Lead dust and lead paint chips

consumed by children in these units can lead to

illness, permanent retardation and in extreme cases,

even death.  Thus measures are being taken to remove

these sources in HUD-subsidized housing built prior to

1978.  Through the guidance provided in this chapter,

owners and managers will be introduced to the testing

and abatement requirements established by the

Lead-Based Paint Poisoning Prevention Act ("the Act")

of 1970, as amended (42 U.S.C. 4822) and the final

regulations published on January 15, 1987.  Nothing in

this section is intended to relieve the owner from

compliance with State and local laws, codes, ordinances

or regulations.

NOTE:  Certain matters set forth in the text below

including references to defective paint and chewable

surfaces have been amended by statute, but not as yet

by regulation.  As regulations are amended, handbook

language will be modified to reflect the change in

regulation.  In addition, references to an elevated

blood level (EBL) which currently stands at 25 ug/dl

will be supplanted in the future by the lower standard

of between 10 and 25 ug/dl established by the Center

for Disease Control.  It is important that owners keep

these aspects in mind when planning for long term

environmental activities.  This Handbook will be

modified as changes concerning the aforementioned

matters evolve.

____________________________________________________________________

19-1                         11/92

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

19-2.     APPLICABILITY.

A.   These requirements will apply to projects

receiving Section 8 Housing Assistance Payments

under the following programs.  Exemptions from

these requirements are listed in 19-2(B):

1.   Substantial Rehabilitation (24 CFR Part 881)

2.   State Agency Substantial Rehabilitation

(24 CFR Part 883)

3.   Loan Management Set Aside (24 CFR Part 886)

B.   The following units in the aforementioned programs

are exempt from compliance with the Act:

1.   0-bedroom units;

2.   elderly or handicapped units (except for

units housing children under seven years of

age); and

3.   units constructed after 1978.

19-3.     PROGRAM REQUIREMENTS.  Different sets of requirements

apply to each specific program area.  What is required

of each unit is based on the type of Section 8

assistance.  If an owner chooses, he may forego the

required inspection and testing of units outlined below

and abate all applicable surfaces in accordance with

the methods set forth in Section 19-6.

A.   Substantial Rehabilitation (Parts 881 and 883).

1.   Units Proposed for Assistance under a HAP

Contract.  For units proposed for assistance

under a HAP contract, the owner is required

to certify all units proposed for assistance

have been inspected for defective paint and

abated in accordance with the guidelines

outlined in 19-6 for defective paint

surfaces, prior to the execution of the HAP

contract.  Defective paint surfaces are those

which have cracking, scaling, chipping,

peeling or loose paint.

____________________________________________________________________

11/92                            19-2

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

2.   Units Currently Covered by a HAP Contract.

As a part of the ongoing maintenance practice

of a property which is the subject of a HAP

contract, the owner should assure that units

are inspected for defective paint.  If

defective paint is present the owner will be

required to abate the defective surface in

accordance with guidelines established in

19-6.

B.   Loan Management Set Aside (LMSA) (Part 886,

Subpart A).

1.   For Units Currently Covered by a HAP

Contract.  As a part of the ongoing

maintenance practice of a property which is

the subject of a HAP contract, the owner

should assure that units are inspected for

defective paint.  If defective paint is

present the owner will be required to abate

the defective surface in accordance with

guidelines established in 19-6.

2.   Units Proposed for Assistance under a HAP

Contract.  The owner is required to certify

all units, which were constructed prior to

1978 and are proposed for assistance under

the LMSA program, have been inspected for

defective paint surfaces and abated according

to the guidelines established for defective

paint surfaces in 19-6.  These are surfaces

on units which have cracking, scaling,

chipping, peeling or loose paint.

For new HAP contracts the following

requirements also apply:

a.   A random sample of all dwelling units

shall be tested for lead-based paint in

chewable surfaces in accordance with the

guidelines established in 19-5.

Chewable surfaces are defined as

protruding painted surfaces up to five

feet from the floor or ground which are

readily accessible to children under

seven years of age.  These surfaces

include, but are not limited to,

protruding corners, window sills and

frames, doors and other protruding

woodwork.

____________________________________________________________________

19-3                         11/92

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

i.   For projects with twenty or more

units, ten units shall be tested.

ii.  For projects with fewer than twenty

units, six units shall be tested.

iii. A sampling of common areas and

exterior surfaces frequently used

by children under 7 years of age,

such as play grounds and day care

centers, shall also be tested.

If all of the units tested in the sample

are found to be lead free, then the

project may also be considered lead free

and no further testing or abatement will

be necessary.  If lead is found in any

of the tested units, all units proposed

for assistance in the project must be

tested.  If lead is found to be present

in any of the common areas, then all

common areas must be tested.

3.   Requirement to abate.  If lead based paint is

discovered in any assisted units or units

proposed for assistance, abatement will be

required under the following conditions:

a.   For new applications.  For the approval

of new units under LMSA, the owner is

required to:

i.   inspect for and abate defective

surfaces (see 19-6 for required

abatement procedures);

ii.  conduct the random sampling test

(described above), in accordance

with the testing requirements of

19-5; and

iii. abate the entire interior and/or

exterior chewable surfaces of all

units proposed for assistance (if

random sampling test results

indicate the presence of lead-based

paint in the unit).

____________________________________________________________________

11/92                          19-4

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

The owner and testing entity must

certify that a unit is free of lead-based

paint, as a condition prior to the

execution of a HAP contract.

b.   In the case of an EBL child.  If an

owner is presented with test results

that a child living in an assisted unit

has an EBL as defined in 19-4, and the

unit is tested positive for lead-based

paint then the owner must abate the unit

according to procedures set forth in 19-6

or relocate the family to a unit free

of lead-based paint.

19-4.     Elevated Blood Level (EBL) Child.  If an owner is

presented with test results that indicate a child seven

years or younger living in a unit has an elevated blood

level (25 micrograms per deciliter of blood or greater

unless otherwise specified by HUD) then the owner is

required to test the unit and if the test is positive

for lead based paint, abate the unit following the

guidelines listed in 19-6, or the owner may forego

testing and abate all unit surfaces.

19-5.     TESTING FOR LEAD-BASED PAINT.  The owner will be

responsible for obtaining testing services.  Testing

must be performed by a State or local health or housing

agency; or an inspector certified or regulated by the

State or local health or housing agency; or an

organization recognized by HUD.  The owner is permitted

to forego all testing requirements and abate all units

without testing if they so choose.

A.   Acceptable methods of testing.  Testing may be

performed using an X-ray Fluorescence Analyzer

(XRF) or other method approved by HUD.  Any

reading above 1 mg/cm2 will be considered a

positive test for lead paint.  If a positive

reading for lead-based paint is questionable, it

must be confirmed by laboratory testing.

B.   Testing Certification.  The testing entity shall

certify the results of the test and a copy will be

available for review by the HUD Field Office.  In

certifying that the unit is free of lead-based

paint for the purpose of Housing Quality Standards

(HQS), a copy of the testing entity's

certification should be attached to the owner's

certification as proof of testing.  Loan

____________________________________________________________________

19-5                         11/92

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

Management Staff should maintain certifications

and document all actions taken concerning

environmental issues in the project file for the

term of the mortgage or if not HUD-insured or

HUD-held, for the term of the HAP contract.

19-6.     ABATEMENT OF LEAD-BASED PAINT.  If the owner is

required or chooses to abate lead-based paint the

following guidelines (which are set forth in 24 CFR 35)

must be followed.  Lead-based paint is classified as a

toxic waste by the Environmental Protection Agency

(EPA) and thus can be more dangerous during and after

removal if the proper procedures are not followed.  At

a minimum, abatement will consist of abating chewable

surfaces, unless otherwise specified by law.  Chewable

surfaces are defined as protruding painted surfaces up

to five feet from the floor or ground which are readily

accessible to children under seven years of age.

A.   Allowable methods of Abatement.  The following

methods of abatement are permitted by HUD:

1.   Covering the existing surface.  Covering the

existing surface is allowed because it

reduces lead dust often generated in the

removal of lead-based paint.  Covering shall

be a sturdy permanent binding that cannot be

removed or damaged by children.  The

following methods are permitted:

a.   Adding a layer of wallboard to the wall

surface.

b.   Wallcoverings which are permanently

attached.

c.   Covering or replacing trim surfaces.

2.   Removal of Lead-based Paint.  Removal is

recommended as a more permanent solution to

the problem.

a.   The following methods of removal are

permitted:

i.   Scraping

ii.  Heat Treatment (infra red or coil

type heat guns)

iii. Chemical Removal

____________________________________________________________________

11/92                          19-6

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

b.   The following methods of removal are

prohibited:

i.   Machine Sanding

ii.  Use of propane or gasoline torches

iii. Washing and repainting the surface

without thorough removal or

covering.

3.   Defective Paint Surfaces.  In the case of

defective paint surfaces, scraping and

repainting the defective paint area will be

considered adequate treatment.

B.   Worker Protection during abatement.  It is

recommended the owner consider requiring

contractors to adhere to the Occupational Safety

and Health Administration (OSHA) guidelines

established for the lead industry.  These

requirements are detailed in 29 CFR 1910.1025 and

will need to be modified for the abatement

industry.  For additional guidance in this matter

it will be helpful to review Lead-Based Paint:

Interim Guidelines for Hazard Identification and

Abatement in Public and Indian Housing and to

contact your local OSHA office.  Major provisions

of 29 CFR 1910.1025 include:

1.   Engineering controls and good work practices;

2.   Medical surveillance and provisions for

medical removal;

3.   Protective clothing and equipment;

4.   Respiratory protection program;

5.   Exposure monitoring;

6.   Record keeping;

7.   Hygiene facilities and practices; and

8.   Training.

C.   Tenant protection during abatement.  It is

important that the following minimum steps be

taken to assure tenant safety as well as the

protection of management during this process.

1.   All tenants in the project should be notified

of the fact that abatement is taking place,

where the abatement is occurring and the

dangers of entering the worksite area or

allowing children to play near the area.

____________________________________________________________________

19-7                         11/92

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

2.   Tenants and furniture should be removed from

the abatement area prior to the abatement by

the removal of LBP.  This precaution is

necessary due to lead dust that may be

present and effects that this dust can have

now and in the future on the health and

safety of the tenants.

3.   If the covering of LBP is used as the method

of abatement, then the owner should take into

consideration the health and safety of the

tenants and the future consequences when

deciding whether or not to remove the tenant

and their possessions from the abatement

area.

D.   State and Local Laws.  Owners and managers are

responsible for adhering to all State and local

laws regarding the testing, abatement and disposal

process.

19-7.     DEVELOPING AN ABATEMENT STRATEGY.  While the

development of an abatement strategy prior to

proceeding with the testing and abatement process is

not required by HUD, it is highly recommended due to

the expense and complexity of the process.  Below is a

listing of several points that are important to

consider in developing an abatement plan:

A.   Outline the Roles and Responsibilities of all

parties involved.

B.   Set forth timetables for testing and abatement to

occur.

C.   Establish unit priorities for abatement.  For

example:

1.   Units with EBL children

2.   Units with children under the age of 7

3.   Units which are designated for families

4.   Units which house the elderly or the

handicapped with no children, but which

become vacant, and a family with children

under the age of seven are in need of such a

unit.

D.   Discuss funding sources.

____________________________________________________________________

11/92                          19-8

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

19-8.     FUNDING TESTING AND ABATEMENT PROCESSES.  While testing

for LBP is an allowable project expense, abatement

funding is not an allowable project expense and is

ineligible for funding with Reserve For Replacement

funds, the Residual Receipts fund is an allowable

source of funding for abatement as well as testing.

With the permission of HUD, the owner may utilize

Residual Receipts for funding testing and abatement

activities.  Grants and loans from state and local

governments should be considered as possible funding

sources for testing and abatement.  Loans may be

repayable with project income if HUD believes the

project is able to support the debt service and

permission is obtained from the first mortgagee.

19-9.     FLEXIBLE SUBSIDY REQUIREMENTS.  When an owner chooses

to apply for funding for the purpose of abating LBP

under the Capital Improvement Loan Program, they are

required to test and certify to testing as outlined in

paragraph 19-5.  Testing may be paid for out of project

income, Residual Receipts or owner funds and the owner

must certify, for each unit where abatement funds are

requested, that the test results indicate a positive

test for LBP as defined in paragraph 19-5.

19-10.    LEGAL EXPENSES - This cost is considered a project

expense only after the owner has fully complied with

HUD's requirements and has been successful in their

defense against the action.

19-11.    OTHER INFORMATIONAL RESOURCES.  While private owners of

HUD assisted properties are not required to adhere to

the HUD guidelines developed for Public and Indian

Housing Authorities, they are a good resource to review

prior to the initiation of testing and abatement.

Lead-Based Paint:  Interim Guidelines for Hazard

Identification and Abatement in Public and Indian

Housing is available by request from your local HUD

Field Office.

____________________________________________________________________

19-9                         11/92

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

SECTION 2:  ASBESTOS

19-10.    Owners of HUD Insured and Assisted properties are

required to comply with EPA and OSHA requirements

concerning the management/abatement (e.g. maintenance,

removal, disposal and encapsulation) of asbestos when

an owner takes an action, such as renovation or

demolition, which involves the disturbance of asbestos

or presence of damaged friable asbestos.  This

requirement applies whether the work requires HUD

approval or not.  This can include capital

improvements, repairs to asbestos enclosed areas,

substantial or moderate rehabilitation (if the asbestos

is located in the area being rehabilitated).  It is

also important that State and local laws be complied

with at all times, and in case of conflict, the more

stringent law shall prevail.  Any abatement that is

undertaken should only be done with EPA trained and

accredited contractors who are experienced in working

with asbestos and knowledgeable of federal, state and

local requirements.

SECTION 3:  POLYCHLORINATED BIPHENYLS (PCBs)

19-13.    HUD Insured and Assisted properties are required to

comply with the EPA's regulation regarding PCBs.  This

is a toxic substance formerly used in electrical

equipment which is currently prohibited by EPA for

installation in apartment buildings.  The law requires

owners to have transformers inspected and replaced.  An

employee of the utility company can inspect the

transformers at the owners request and inform the

owner/manager of any evidence of PCBs in the electrical

system.  Under the Toxic Substances Control Act and

implementing EPA regulations, PCBs are required to be

removed and replaced.

____________________________________________________________________

11/92                          19-10

_____________________________________________________________________

4350.1 REV-1

CHG-2

____________________________________________________________________

SECTION 4:  UNDERGROUND STORAGE TANKS (USTs)

19-14.    HUD does not have any specific requirements regulating

USTs in existing properties.  Project owners should be

aware that they are subject to the EPA regulations for

reporting, testing, replacement, etc. outlined in 40

CFR Part 280 and 281 or if State requirements are more

stringent, owners will be subject to those.  Typically,

EPA and State regulations were intended to address

environmental problems relating primarily to old and

new service station sites and industrial chemical

storage sites.  Tanks used for storing heating oil for

consumptive use on the premises are normally exempt

under the regulations.  For additional guidance the

owner should contact the Field Office Loan Management

Branch.

_________________________________________________________________

19-11                        11/92